Slave axis homing note

- Set the a axis as x axis slave.

In the menu Config ->slave axis

- Enable a home in the mach3 input io config

Config ->ports and pins->input signals:

- Connect the a home switch like x,y,z  home switch. Then click Refer all home

Mach3 will auto refer salve axis a home.

When disable a home function.the slave  a axis will not home.